MIT Assignment-10

Jeetu Mehra

U24AI032

• Given are two numbera. Using 8086 instruction set

Q1:

Add, substract the given numbers CODE:

; You may customize this and other start-up templates;

; The location of this template is c:\emu8086\inc\0_com_template.txt

org 100h

MOV AL, 25H      ; 1st number

MOV BL, 15H      ; 2ND number

; ADD

MOV AH, AL

ADD AH, BL       ; AH = AL + BL

; SUB

MOV CL, AL

SUB CL, BL       ; CL = AL - BL

ret

OUTPUT:

Q2:

Convert the numbers into ASCII

CODE:

org 100h

MOV AL, 25       ; Number to convert (decimal 25) -> ASCII

MOV BL, 10       ; Divisor

;  Convert number in AL to ASCII

XOR AH, AH       ; Clear AH for DIV

DIV BL           ; AL / 10 -> Quotient in AL, Remainder in AH

ADD AL, '0'      ; Convert quotient to ASCII, stored in AL

ADD AH, '0'      ; Convert remainder to ASCII, stored in AH

; Now:

; AL = ASCII of tens digit

; AH = ASCII of units digit

ret

OUTPUT:

Q3:

Add 47H to output of addition. If a carry is generated then display FFH else display 00H in register CX.

CODE:

org 100h

MOV AL, 25H      ; 1st number

MOV BL, 15H      ; 2nd number

; ADD

MOV AH, AL

ADD AH, BL       ; AH = AL + BL

; --- Add 47H and check carry ---

MOV AL, AH       ; Load addition result

ADD AL, 47H      ; Add 47H

JC  carry_flag   ; Jump if carry generated

; --- No carry ---

MOV CX, 0000H    ; Set CX = 0000H if no carry

JMP done

carry_flag:

MOV CX, 0FFFFH   ; Set CX = FFFFH (FFH) if carry

done:

ret

ret

OUTPUT: