MIT Assignment-08

Jeetu Mehra

U24AI032

Q1.Write a program to add the following data bytes stored in memory locations starting at XX60H and display the sum at the output port if the sum does not generate a carry. If a result generates a carry, stop the addition, and display 01H at the output port.

Data (H):

First Set: 37, A2, 14, 78, 97

Second Set: 12, 1B, 39, 42, 07

LXI H, 2660H

MVI M, 37H

INX H

MVI M, 0A2H

INX H

MVI M, 14H

INX H

MVI M, 78H

INX H

MVI M, 97H

INX H

MVI M, 12H

INX H

MVI M, 1BH

INX H

MVI M, 39H

INX H

MVI M, 42H

INX H

MVI M, 07H

; -------- ADD LOGIC --------

LXI H, 2660H

MVI B, 0AH

MVI A, 00H

LOOP: ADD M

JC CARRY

INX H

## DCR B

JNZ LOOP

# OUT 01H

HLT

CARRY: MVI A, 01H

OUT 01H

HLT

Q2.In Assignment 20, modify the program to count the number of data bytes that have been added and display the count at the second output port.

LXI H, 2660H

MVI M, 37H

INX H

MVI M, 0A2H

INX H

MVI M, 14H

INX H

MVI M, 78H

INX H

MVI M, 97H

INX H

MVI M, 12H

INX H

MVI M, 1BH

INX H

MVI M, 39H

INX H

MVI M, 42H

INX H

MVI M, 07H

; -------- ADD + COUNT --------

LXI H, 2660H

MVI B, 0AH

MVI A, 00H

MVI C, 00H

LOOP: ADD M

JC CARRY

INR C

INX H

DCR B

JNZ LOOP

OUT 01H

MOV A, C

OUT 02H

HLT

CARRY: MVI A, 01H

OUT 01H

MOV A, C

# OUT 02H

### HLT

Q3.The temperatures of two furnaces are being monitored by a microcomputer. A set of five readings of the first furnace, recorded by five thermal sensors, is stored at the memory location starting at XX50H. A corresponding set of five readings from the second furnace is stored at the memory location starting at XX60H. Each reading from the first set is expected to be higher than the corresponding reading from the second set. For example, the temperature reading at location 54H (T₅₄) is expected to be higher than the temperature reading at location 64H (T₆₄). Write a program to check whether each reading from the first set is higher than the corresponding reading from the second set.

- If all readings from the first set are higher → turn ON bit D₀ of output PORT1

- If any one reading is lower → stop and output FF at PORT1 (emergency signal)

Data (H):

First Set: 82, 89, 78, 8A, 8F

Second Set: 71, 78, 79, 82, 7F

LXI H, 2650H

MVI M, 45H

INX H

MVI M, 50H

INX H

MVI M, 60H

INX H

MVI M, 70H

INX H

MVI M, 80H

; -------- STORE SECOND SET --------

LXI H, 2660H

MVI M, 30H

INX H

MVI M, 40H

INX H

MVI M, 55H

INX H

MVI M, 65H

INX H

MVI M, 75H

; -------- COMPARISON --------

LXI H, 2650H

LXI D, 2660H

MVI B, 05H

LOOP: MOV A, M

MOV C, A

LDAX D

CMP C

JNC FAIL

INX H

INX D

DCR B

JNZ LOOP

MVI A, 01H

OUT 01H

HLT

FAIL: MVI A, 0FFH

OUT 01H

HLT

Q4.A set of eight data bytes is stored in memory locations starting from XX70H.

Write a program to subtract two bytes at a time and store the result in a sequential order in memory locations starting from XX70H.

Data (H):

F9, 38, A7, 56, 98, A2, F4, 67

LXI H, 2670H

MVI M, 0F9H

INX H

MVI M, 38H

INX H

MVI M, 0A7H

INX H

MVI M, 56H

INX H

MVI M, 98H

INX H

MVI M, 0A2H

INX H

MVI M, 0F4H

INX H

MVI M, 67H

; -------- SUBTRACTION --------

LXI H,2670H

LXI D,2670H

MVI C,04H

LOOP: MOV A,M

INX H

MOV B,M

SUB B

STAX D

INX D                INX H

DCR C

JNZ LOOP

HLT

Q5.A set of eight data bytes is stored in the memory location starting at XX50H. Check each data byte for bits D₇ and D₀.

- If D₇ or D₀ is 1 → reject the data byte

- Otherwise → store the data bytes at memory locations starting at XX60H

Data (H):

80, 52, E8, 78, F2, 67, 35, 62

; Assume XX = 20 → so addresses are 2050H and 2060H

LXI H, 2650H     ; Point to source memory

MVI A, 80H

MOV M, A

INX H

MVI A, 52H

MOV M, A

INX H

MVI A, 0E8H

MOV M, A

INX H

MVI A, 78H

MOV M, A

INX H

MVI A, 0F2H

MOV M, A

INX H

MVI A, 67H

MOV M, A

INX H

MVI A, 35H

MOV M, A

INX H

MVI A, 62H

MOV M, A

LXI H, 2650H     ; Source pointer

LXI D, 2660H     ; Destination pointer

MVI C, 08H       ; Counter

LOOP: MOV A, M        ; Load data byte

ANI 81H         ; Check D7 and D0

JNZ SKIP        ; If either is 1 → reject       MOV A, M        ; Reload original data

STAX D          ; Store at destination

INX D           ; Increment destination pointer

SKIP: INX H           ; Next source byte

DCR C

JNZ LOOP

HLT

Q6.A microcomputer is designed to control various appliances and lights in your house.

The system has an output port with address 01H, and various units are connected to bits D₇ to D₀.

On a cool morning you want to turn ON:

- Radio

- Coffeepot

- Space heater

Write appropriate instructions for the microcomputer. Assume R/W memory begins at 2000H.

LXI SP, 2099H   ; Initialize Stack Pointer

MVI A, 70H      ; Load 70H (Radio + Coffeepot + Space Heater ON)

OUT 01H         ; Send to Output Port 01H

HLT             ; Stop

An additional input port with eight switches and address 01H is connected to the microcomputer to control the same appliances and lights from both bedroom and kitchen.

Write instructions to turn ON devices from any of the input ports.

LXI SP, 2099H   ; Initialize Stack Pointer

IN 00H          ; Read Kitchen Input Port (00H) into Accumulator

MOV B, A        ; Save kitchen input in register B

IN 01H          ; Read Bedroom Input Port (01H) into Accumulator

ORA B           ; OR bedroom with kitchen inputs

; (device ON if either port requests it)

OUT 01H         ; Send combined result to Output Port 01H

HLT             ; Stop

```

---

## Explanation of Logic

```

Kitchen  (00H): 1 0 1 0 0 1 0 1  → A5H

Bedroom  (01H): 1 0 0 1 0 0 0 1  → 91H

OR Result     : 1 0 1 1 0 1 0 1  → B5H

↑    ↑ ↑

A/C  Coffepot+Radio ON from either room

Q7.If the switch S₇ of input PORT0 (connected to data line D₇) is at logic 1 and all other switches  are at logic 0, specify the contents of the accumulator when the instruction IN PORT0 is executed.

MVI A, 9FH

IN PORT0

MOV B, A

OUT PORT1

HLT

Accumulator (A) = 80H Explanation:

Only switch S7 is ON, so D7 = 1 and all other bits are 0.

So input from PORT0 = 10000000 = 80H.

When IN PORT0 is executed, accumulator gets this value. Hence A = 80H.

Q8.Specify the output at PORT1 and the contents of register B after executing the instructions in Assignment 6.

PORT1 = 80H

Register B = 80H Explanation:

From Q6, input from PORT0 = 80H (only D7 = 1).

IN PORT0 loads A = 80H.

MOV B, A copies A to B, so B = 80H.

OUT PORT1 sends A to PORT1, so PORT1 = 80H.