|  | MIT Assignment-08 Jeetu Mehra U24AI032 |
| --- | --- |
| 1 | Write a Program to count continuously in hexadecimal from FFH to 00H in a system with 0.5μs cock period. Use register C to set up a one milisecond delay between each count and display the numbers at one of the output port. MVI A, 00H LOOP: OUT 01H CALL DELAY DCR A CPI 0FFH JNZ LOOP OUT 01H HLT DELAY: MVI C, 00H D1: NOP NOP DCR C JNZ D1 RET |

| 2 | Write a program to count from 0 to 9 with a one second delay between each count. At the count of 9,the counter should reset itself to 0 and repeat sequence continuously. Use register pair HL to set up the delay,and display each count at one of the output port. Assume the clock frequency of the microprocessor is 1 MHz. ; Program to count 0-9 with 1 second delay ; Clock Frequency: 1 MHz ; Port: 01H (Assumed) ORG 0000H START: MVI C, 00H ; Initialize register C as the counter (0) DISPLAY:MOV A, C ; Move the current count to Accumulator OUT 01H ; Output the count to Port 01H ; --- 1 Second Delay Subroutine using HL Pair --- LXI H, A2C2H ; Load HL with A2C2H (calculated for 1s) DELAY: DCX H ; Decrement HL pair (6 T-states) MOV A, L ; Move L to A (4 T-states) ORA H ; OR with H to check if HL = 0 (4 T-states) JNZ DELAY ; If not zero, jump back to DELAY (10 T-states) ; ----------------------------------------------- INR C ; Increment the counter CPI 0AH ; Compare Accumulator with 10 (0AH) MOV A, C ; Move C to A for comparison CPI 0AH ; Check if count has reached 10 JZ START ; If count is 10, reset to 0 JMP DISPLAY ; Otherwise, display the next number |
| --- | --- |

| 3 | Given: Clock period = 0.33 μs LXI B,12FFH → 12FFH = 4863 (loop count) T-states per iteration: DCX B (6) + XTHL (16) + XTHL (16) + NOP (4) + NOP (4) + MOV A,C (4) + ORA B (4) + JNZ (10) = 64 T-states Last iteration: JNZ = 7 instead of 10 → 64 - 3 = 61 T-states Total T-states: = (4862 × 64) + 61 + 10 = 311168 + 61 + 10 = 311239 T-states Total delay: = 311239 × 0.33 μs ≈ 102709 μs ≈ 102.7 ms Final Answer: 102.7 ms |
| --- | --- |

| 4 | Given: Clock period = 0.33 μs LXI B,2384H → 2384H = 9092 (loop count) T-states: LXI B = 10 DCX B = 6 MOV A,C = 4 ORA B = 4 JNZ = 10 (jump), 7 (last) T-states per iteration: = 6 + 4 + 4 + 10 = 24 T-states Last iteration: = 24 - 3 = 21 T-states Total T-states: = (9091 × 24) + 21 + 10 = 218184 + 21 + 10 = 218215 T-states Total delay: = 218215 × 0.33 μs ≈ 72010 μs ≈ 72.0 ms Final Answer: 72.0 ms |
| --- | --- |
| 5 | A number is stored in memory location XX50H .Write a program to 1. Transfer the number from memory locations to A 2. Store a result at output port 02H LXI H,2650H MOV A,M |

OUT 02H

HLT

|  | Write a subroutine to - To find 2’s complement of a number stored in A - Return COMP: CMA INR A RET |
| --- | --- |

| 6 | . Write a program to - Take number from input pot 1 and Transfer the numbers to A - Take another a number from memory location 2000H. 3. Store a result at output port 02H MVI A,34H STA 2000H IN 01H MOV B,A LDA 2000H ADD B OUT 02H HLT |
| --- | --- |

|  | Write a subroutine to 1. Add both numbers 2. Return . MVI A,34H STA 2000H IN 01H MOV B,A LDA 2000H CALL ADDR OUT 02H HLT ADDR: ADD B RET |
| --- | --- |

| 7 | - SP after line 1 SP = 0400H - First data byte stored location - PUSH decreases SP first SP → 03FFH (first location used) - Value at 03FEH after PUSH H H = 22H, L = FFH Execution: - SP → 03FF → store H (22H) - SP → 03FE → store L (FFH) Memory[03FEH] = FFH - After PUSH B B = 20H, C = 55H Current SP = 03FEH Execution: - SP → 03FD → store B (20H) - SP → 03FCH → store C (55H) SP = 03FCH Memory[03FDH] = 20H |
| --- | --- |

|  | (e) After POP H (line 20) Stack currently: Address Value 03FC 55 |  |
| --- | --- | --- |
|  | 03FD | 20 |
|  | 03FE | FF |
|  | 03FF | 22 |
|  | POP H: - L ← 55 (from 03FC) - H ← 20 (from 03FD) HL = 2055H |  |
| 8 | A multiplicand is stored in memory location XX50H and a multiplier is stored in location XX51H. Write a program to - Transfer the two numbers from memory loations to HL pair - Store a product at output port 02H MVI A,05H STA 2650H MVI A,03H STA 2651H LHLD 2650H MOV B,L MOV C,H MVI A,00H |  |

|  | LOOP: ADD B DCR C JNZ LOOP OUT 02H HLT |
| --- | --- |

|  | Write a subroutine to - Multiply numbers stored into HL pair - Return the result to HL pair MVI A,05H |
| --- | --- |

|  | STA 2650H MVI A,03H STA 2651H LHLD 2650H CALL MUL MOV A,L OUT 02H HLT MUL: MOV B,L MOV C,H MVI A,00H LOOP: ADD B DCR C JNZ LOOP MOV L,A MVI H,00H RET |
| --- | --- |

9

(

a) After CALL execution

Initial:

SP = 20CDH

Return Address = 200BH

→ High = 20H, Low = 0BH

Execution:

SP → 20CCH → store 20H

SP → 20CBH → store 0BH

Answer:

(20

CCH) = 20H

CBH) = 0BH

(20

PC = 2060H

SP = 20CBH

(

b) After PUSH H and PUSH B

PUSH H (H=00, L=00)

SP → 20CA → store H (00)

SP → 20C9 → store L (00)

PUSH B (B=0F, C=?? assume 00)

SP → 20C8 → store B (0F)

SP → 20C7 → store C (00)

Stack:

20

CCH → 20H

20

CBH → 0BH

20

CAH → 00H

20

C9H → 00H

20

C8H → 0FH

20

C7H → 00H

|  | (c) SP after PUSH B (line 2061H) SP = 20C7H |  |
| --- | --- | --- |
|  | (d) After RET POP sequence: - PCL ← 0BH - PCH ← 20H SP moves: 20C7 → 20C8 → 20C9 → 20CA → 20CB → 20CC → 20CD Final SP = 20CDH |  |